AI Sign-to-Speech

Smart Gloves System

Phase 2 — Processing Pipeline

Filter + Normalize + Processing Thread

Who this document is for: Every team member — hardware engineers, software developers, and ML engineers. It explains what Phase 2 builds, why every decision was made, how the three new files connect together, and what the completed pipeline looks like. Read this before touching any Phase 2 code.

# 1. What Phase 2 Is and Why It Exists

After Phase 1, the Python pipeline receives raw sensor frames from the ESP32 over USB Serial. These frames contain raw ADC integers — numbers like 2134, 2048, 1650. These numbers are hardware-specific. They depend on the exact magnet placement, the specific SS49E sensor, the temperature of the room, and how worn the glove is.

The core problem: the BiLSTM machine learning model cannot train on raw ADC values. A reading of 2134 means something completely different on a different sensor, a different day, or a different person's hand. The model needs hardware-independent values — numbers that mean the same thing regardless of which glove or which session produced them.

Phase 2 solves this by adding a transformation layer between the serial pipeline and everything above it. Think of it like a currency converter — the underlying physical reality (how bent is this finger?) is the same, but Phase 2 expresses it in a universal language that the model can actually learn from.

Phase 1 output (raw): R_Thumb = 2134 (hardware-specific, meaningless to ML) Phase 2 output (processed): R_Thumb = 0.73 (universal: finger is 73% bent)

## The Three Files Phase 2 Builds

| File | What It Does | Why It Exists |
| --- | --- | --- |
| processing/filter.py | Smooths noisy ADC readings using EMA | Hall sensors produce random electrical noise even when finger is still |
| processing/calibration.py | Maps each sensor's personal ADC range to 0.0-1.0 | Every sensor has slightly different characteristics — calibration makes them speak a universal language |
| processing/processing_thread.py | Consumes Queue, applies filter + calibration, emits clean frames via Qt Signal | Processing must run on a separate thread so the UI never stutters |

# 2. filter.py — The Noise Smoother

Hall sensors are never perfectly quiet. Even when your finger is completely still, the ADC reading fluctuates randomly by 10 to 20 counts. This is called ADC noise — random electrical fluctuation that has nothing to do with real finger movement.

The analogy: imagine trying to draw a straight horizontal line but your hand trembles slightly. The line comes out jagged even though your intent was perfectly straight. Your Hall sensor has the same problem — it trembles electrically even when physically still. The EMA filter smooths those trembles away.

## The EMA Formula

EMA stands for Exponential Moving Average. The formula is:

filtered = alpha * new_value + (1 - alpha) * previous_filtered With alpha = 0.25: filtered = 0.25 * new_value + 0.75 * previous_filtered

The intuition: instead of trusting the new reading completely, you take a weighted blend of the new reading and what you already believed. If one reading spikes randomly (noise), it only moves your belief by 25% of that spike. But if the finger genuinely moves, every new reading consistently pushes the belief in the same direction, so it tracks real movement within 3 to 4 frames.

## Why Alpha = 0.25

| Alpha Value | Behaviour | Problem |
| --- | --- | --- |
| 1.0 | No smoothing — raw signal | Too noisy for ML |
| 0.5 | Light smoothing | Still somewhat noisy |
| 0.25 (chosen) | Balanced noise vs responsiveness | None — this is the correct value |
| 0.1 | Heavy smoothing | Too sluggish — lags behind real motion |

At 30 Hz, noise spikes are random and short-lived. Real finger movement is sustained across many frames. Alpha 0.25 exploits exactly that difference — noise is absorbed, real movement passes through.

## Why Filter BEFORE Normalizing — Not After

Critical rule: always filter raw ADC integers first, then normalize the smooth result. If you normalize first and filter second, the filter operates on 0.0-1.0 values and small noise near the boundaries causes clipping artifacts — values get stuck at 0.0 or 1.0 incorrectly. Filter on raw ADC where noise is only 1% of the range. Then normalize the clean result.

## Why a Class, Not a Function

EMA requires remembering the previous filtered value between calls. A Python function has no memory between calls. A class holds state cleanly per instance. Each of the 16 sensor channels gets its own EMAFilter instance — the thumb filter's state never touches the index finger filter's state.

# 16 independent filter instances — one per sensor channel self.filters = { hand: {ch: EMAFilter() for ch in FINGER_CHANNELS + IMU_CHANNELS} for hand in ['right', 'left'] }

## The reset() Method

The reset() method clears the filter's previous_filtered value back to None. This must be called whenever the serial connection drops and reconnects, or when a new calibration profile is loaded. If you don't reset, the first frames of the new session blend with stale data from the previous session, producing incorrect normalized values at the start of a gesture recording.

# 3. calibration.py — The Universal Translator

Even after filtering, raw ADC values are still hardware-specific. Your thumb sensor reads 2900 when open and 1650 when closed. Your index finger sensor reads 2800 when open and 1700 when closed. Different ranges for the same physical motion.

The analogy: imagine two thermometers — one in Celsius, one in Fahrenheit. 37 means very different things on each. If you mix readings from both into one dataset, the model is confused by the same physical temperature having different numbers. Calibration is the process of translating every sensor to the same universal scale.

## The Normalization Formula

bend = (filtered_value - min_value) / (max_value - min_value) With your hardware (open = 2900, closed = 1650): min_value = 2900 (open hand — recorded first in wizard) max_value = 1650 (closed hand — recorded second) Open hand: (2900 - 2900) / (1650 - 2900) = 0 / -1250 = 0.0 Closed hand: (1650 - 2900) / (1650 - 2900) = -1250 / -1250 = 1.0 Halfway: (2275 - 2900) / (1650 - 2900) = -625 / -1250 = 0.5

Notice that min_value is larger than max_value for your sensor. This is because the magnet moves away from the sensor as the finger bends, so ADC decreases as bend increases. The formula handles this correctly — negative divided by negative gives positive. The calibration wizard just needs to record open-hand first and closed-hand second. The math takes care of the inverted direction automatically.

## The Dummy Channel Problem — Division by Zero

With only one real sensor in Phase 2, all other 15 channels send a constant 2048 ADC. During calibration, both open and closed positions record 2048 for dummy channels. This creates a division by zero in the normalization formula.

# Without protection: bend = (2048 - 2048) / (2048 - 2048) = 0 / 0 ← Python crash: ZeroDivisionError # The fix — one condition handles both dummy and real channels: if max_val == min_val: return 0.0 # dummy channel — safe output, no division # When full glove arrives — real sensors have different min and max: # min = 2850, max = 1700 → max != min → condition never triggers # No code change needed at transition. Just re-run calibration.

## The Clamping Step

Filtered values can occasionally exceed the calibrated range slightly, especially at the very start of a session before the EMA filter has fully settled. Without clamping, a value like -0.02 or 1.03 could enter the dataset. The BiLSTM trains on values it will never see during real inference — hurting accuracy.

# Clamp to 0.0-1.0 — no exceptions allowed in the dataset return max(0.0, min(1.0, bend)) # Examples: # -0.04 → max(0.0, -0.04) = 0.0 (clamped at floor) # 1.03 → min(1.0, 1.03) = 1.0 (clamped at ceiling)

## Named User Profiles — Why Not Date-Stamped Files

The hardware design uses modular sensor and magnet modules that can be attached and detached. Different team members have different hand sizes. Different hardware configurations have different sensor characteristics. A date-stamped calibration file would not survive a module swap mid-session.

Instead, calibration is saved as named user profile JSON files — anoop.json, teammate1.json, and so on. Each profile stores that person's min and max values permanently. On startup, the user selects their profile. A Recalibrate button is always available for when hardware modules are swapped. This is implemented in the Phase 3 UI calibration wizard.

data/calibration/ ├── anoop.json ← Anoop's personal calibration ├── teammate1.json ← Teammate 1's calibration └── teammate2.json ← Teammate 2's calibration Each file contains: { "min_values": { "right": { "thumb": 2900, "index": 2048, ... } }, "max_values": { "right": { "thumb": 1650, "index": 2048, ... } } }

## Why IMU Values Are NOT Normalized

IMU values (pitch, roll, yaw) are already in degrees — a universal measurement. 12.4 degrees of pitch means the same thing on every sensor, every session, every day. There is nothing to translate. Normalizing degrees would actually lose information — the model benefits from knowing the actual angle, not just where it falls within some arbitrary range. IMU values pass straight through calibration unchanged.

# 4. processing_thread.py — The Pipeline Manager

The analogy: think of a car assembly line. One station welds the frame, the next installs the engine, the next adds the body. Each station does one job and passes the result forward. processing_thread.py is the assembly line manager — it picks up raw frames, runs them through filter.py then calibration.py, checks for drops, and passes clean processed frames to the UI.

## Why a Separate Thread

The UI needs to render smoothly at the same time processing is happening. At 30 Hz, a calculation arrives every 33 milliseconds. If processing ran on the main thread, every filter calculation would block the UI from updating. The UI would stutter constantly. A dedicated processing thread runs independently — the UI renders at full speed and processing happens simultaneously.

## Why QThread Instead of Python threading.Thread

Python's threading.Thread cannot emit Qt Signals safely. Qt Signals are the mechanism for communicating results back to the UI. QThread integrates with PyQt5's signal/slot system — signals emitted from a QThread are automatically routed to the main thread safely. This is PyQt5's built-in thread safety mechanism.

# Python threading.Thread — CANNOT emit Qt Signals safely # QThread — CAN emit Qt Signals, automatically routed to main thread class ProcessingThread(QThread): # correct frame_ready = pyqtSignal(object) # signal carrying processed frame frame_drop_detected = pyqtSignal(int) # signal carrying drop count status_message = pyqtSignal(str) # signal carrying warning text

## Thread Communication — The Two Channels

The processing thread communicates through exactly two channels. It never shares mutable state with other threads directly.

| Direction | Mechanism | Why |
| --- | --- | --- |
| serial_thread → processing_thread | queue.Queue | Thread-safe bounded buffer. Neither thread touches the other's memory directly. |
| processing_thread → main thread (UI) | Qt Signal | PyQt5's built-in thread-safe UI update mechanism. Direct UI access from processing thread would crash or corrupt display. |

## Frame Drop Detection

Every raw frame has a frame_id that counts from 0 to 9999 and then wraps back to 0. The processing thread remembers the last frame_id it saw and checks every new frame against it.

# Calculate expected next frame ID (handles 9999 → 0 rollover) expected_id = (last_frame_id + 1) % (FRAME_ID_MAX + 1) # % 10000 if current_id != expected_id: # Gap detected — frames were dropped # If NOT recording: log warning, keep going # If recording: emit frame_drop_detected signal immediately # gesture recorder discards the sample

Think of it like page numbers in a book. If you are on page 41 and the next page says 45, you know pages 42, 43, and 44 are missing. Frame drops outside recording are just logged as warnings. Frame drops during recording are fatal — the 60-frame gesture window is corrupted and the sample must be discarded immediately.

## Why FRAME_ID_MAX + 1 in the Modulo

config.py defines FRAME_ID_MAX = 9999, which is the maximum value the frame ID reaches. The rollover calculation must use modulo 10000 — not 9999. Using 9999 as the modulo would cause frame 9999 to never be expected correctly, producing a false drop warning every time the counter rolls over.

# WRONG: % 9999 skips frame 9999 # (9999 + 1) % 9999 = 10000 % 9999 = 1 (not 0 as expected) # CORRECT: % 10000 handles rollover properly # (9999 + 1) % 10000 = 10000 % 10000 = 0 (correct) expected_id = (last_frame_id + 1) % (FRAME_ID_MAX + 1)

## The Processing Pipeline Per Frame

Raw Frame (ADC integers from Queue) ¦ v Step 1: Frame drop detection ← check frame_id gap ¦ v Step 2: EMA filter ← smooth all 16 channels ¦ v Step 3: Calibration normalize ← map to 0.0-1.0 ¦ v Step 4: Emit Qt Signal ← send to UI safely

# 5. The Qt Signal System — Thread-Safe UI Updates

Qt Signals are the foundation of how the processing thread communicates with the UI. Understanding them is essential because this pattern is used throughout the entire application from Phase 2 onwards.

The analogy: think of a Qt Signal like a radio broadcast. The processing thread is the radio station — it transmits on a frequency without knowing who is listening. The UI label is a radio receiver tuned to that frequency. When the station broadcasts, every receiver tuned to that frequency gets the message automatically. The station and receiver never meet directly.

In Phase 2 testing, the frame_ready signal is connected to a simple print function. In Phase 3, the same signal is connected to a UI label instead. The processing thread code does not change at all. This is the core power of signals — the sender does not know or care what receives it.

# Phase 2 test — signal connected to print function processing_thread.frame_ready.connect(on_frame_received) def on_frame_received(processed_frame): print(f"R_T: {processed_frame['right']['thumb']:.2f}") # Phase 3 onwards — same signal, different receiver processing_thread.frame_ready.connect(ui_label.update_values) # Processing thread code never changes — only what it connects to changes

## Why QCoreApplication Is Required

QThread and Qt Signals require a Qt application instance to exist before they can work. QCoreApplication is the minimal Qt application — no windows, no UI, just the event loop. In Phase 2 console testing, QCoreApplication is sufficient. In Phase 3, this becomes QApplication which supports UI windows.

## The Ctrl+C Problem and Fix

When the Qt event loop (app.exec_()) is running, it blocks Python from handling Ctrl+C directly. Without a fix, pressing Ctrl+C causes a core dump instead of a clean exit.

# The fix — two parts working together: # Part 1: intercept Ctrl+C and tell Qt to quit cleanly signal.signal(signal.SIGINT, lambda *args: app.quit()) # Part 2: wake Python every 200ms so it can check for Ctrl+C # (Qt event loop blocks Python — timer gives Python a chance to run) timer = QTimer() timer.start(200) timer.timeout.connect(lambda: None)

Think of it like this — Qt is driving the car and Python is a passenger. Without the fix, Ctrl+C yanks the steering wheel and crashes. With the fix, Ctrl+C politely asks the driver to pull over, and Qt stops cleanly.

# 6. The Complete Phase 2 Data Flow

ESP32 firmware ¦ sends: F,0042,2134,2048,2048,...,0.0,0.0,0.0,20533 ¦ at: 30 Hz over USB Serial v serial_thread.py ¦ reads serial port on dedicated thread ¦ validates: prefix check, field count, checksum, type checks ¦ pushes validated Raw Frame dict to Queue v queue.Queue (maxsize=100, ~3.3 seconds buffer) v processing_thread.py ¦ pulls Raw Frame from Queue ¦ checks frame_id gap for drops ¦ applies EMAFilter to all 16 channels ¦ applies Calibrator.normalize_frame() ¦ emits frame_ready Qt Signal v Qt Signal (routed safely to main thread) v on_frame_received() [Phase 2: print function] ¦ prints: 4025 ¦ R_T: 0.73 ¦ R_I: 0.00 ¦ ... ¦ Pitch: 0.0 v [Phase 3+: UI labels, signal plots, 3D skeleton]

# 7. Testing Phase 2

## What Each Test File Verifies

| Test File | Tests | Hardware Needed |
| --- | --- | --- |
| test_filter.py | First call seeding, noise spike absorption, sustained movement tracking, reset behaviour | None — uses fake values |
| test_calibration.py | Open/closed normalization, dummy channel safety, clamping, save/load from disk | None — uses fake values |
| main.py (live) | Full pipeline with real ESP32, real sensor, real normalized output | ESP32 + Hall sensor |

## Phase 2 Completion Criteria

- Open hand reads approximately 0.0 after calibration

- Fully bent finger reads approximately 1.0 after calibration

- EMA smoothing visibly reduces noise compared to Phase 1 raw ADC values

- Dummy channels (R_I, R_M, R_R, R_L) all output 0.00 consistently

- Frame drop during test logs warning correctly

- Ctrl+C exits cleanly with no core dump

- All test_filter.py tests pass with green checkmarks

- All test_calibration.py tests pass with green checkmarks

## What the Console Output Looks Like When Working

Frame ¦ R_T ¦ R_I ¦ R_M ¦ R_R ¦ R_L ¦ Pitch ----------------------------------------------------------------- 4018 ¦ R_T: 0.00 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 4019 ¦ R_T: 0.12 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 4020 ¦ R_T: 0.38 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 4021 ¦ R_T: 0.67 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 4022 ¦ R_T: 0.89 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 4023 ¦ R_T: 1.00 ¦ R_I: 0.00 ¦ R_M: 0.00 ¦ R_R: 0.00 ¦ R_L: 0.00 ¦ Pitch: 0.0 ^^ finger bending from open to closed

# 8. Config.py Constants Added in Phase 2

| Constant | Value | Why This Value |
| --- | --- | --- |
| EMA_ALPHA | 0.25 | Balances noise reduction vs responsiveness at 30 Hz |
| FRAME_ID_MAX | 9999 | Maximum frame ID before rollover to 0 — firmware uses this value |
| CALIBRATION_DIR | 'data/calibration' | Where named user profile JSON files are saved |
| FINGER_CHANNELS | ['thumb','index','middle','ring','little'] | Channel names matching frame dict structure |
| IMU_CHANNELS | ['pitch','roll','yaw'] | IMU channel names — pass through calibration unchanged |

# 9. Key Design Decisions — Why Each Choice Was Made

## Why Filter Then Normalize (Not Normalize Then Filter)

If you normalize first, the filter operates on 0.0-1.0 values. Small noise near 0.0 or 1.0 gets clipped by the clamp() call — you lose information. Worse, the filter's behaviour changes depending on where in the 0-1 range you are. Filter on raw ADC where noise is only 1% of the full range. Then normalize the clean result. This is non-negotiable.

## Why Discard on Frame Drop During Recording

A dropped frame means a gap in the 60-frame sequence. The BiLSTM model trains on the temporal pattern across all 60 frames — it learns how fingers move over time, not just where they end up. A gap corrupts that pattern. Silently saving a corrupted sample poisons the training data. The model learns wrong patterns with no warning and accuracy suffers with no obvious cause. Discarding and re-recording takes 2 seconds. Retraining a bad model takes hours.

## Why Named Profiles Instead of Date-Stamped Files

The hardware uses modular sensor and magnet modules. Swapping a module changes the sensor characteristics and invalidates any previous calibration. Date-stamped files would not survive a module swap mid-session and would not distinguish between different team members using the same glove. Named profiles solve both problems — each person has their own permanent calibration and can recalibrate instantly after a hardware module swap.

## Why the Queue Has 100 Frames Capacity

At 30 Hz, 100 frames represents 3.3 seconds of buffering. This gives the processing thread a comfortable margin if it runs slightly behind the serial thread for brief periods. 100 frames is small enough that memory usage is negligible, but large enough to absorb normal processing jitter without dropping frames.

# 10. What Phase 2 Changes vs Phase 1

| Aspect | Phase 1 | Phase 2 |
| --- | --- | --- |
| Output | Raw ADC integers (2134, 2048...) | Normalized floats (0.73, 0.00...) |
| Threading | Serial thread only | Serial thread + Processing thread |
| Qt usage | None | QCoreApplication + QThread + Signals |
| Calibration | None | Named user profiles, saved as JSON |
| Noise handling | None — raw signal | EMA filter on all 16 channels |
| Frame consumption | main.py reads Queue directly | Processing thread reads Queue |
| UI communication | print() in main thread | Qt Signal to slot function |
| .py files added | None | filter.py, calibration.py, processing_thread.py |

# 11. What Changes When Full Glove Arrives

Every Python file written in Phase 2 is final production code. When the full 3D-printed glove arrives with all 10 sensors, only two things change.

| What Changes | How | Python Files Affected |
| --- | --- | --- |
| ESP32 firmware (.ino) | Replace DUMMY_ADC with real analogRead() for each finger | None |
| Calibration data | Re-run the calibration wizard — real min/max recorded | None |

filter.py, calibration.py, processing_thread.py, serial_thread.py, packet_parser.py — none of these change when the full glove arrives. The pipeline already processes all 16 channels on every frame. Right now, 15 channels receive 2048 and output 0.0. When real sensors arrive, those channels receive real ADC values and output real normalized values. The pipeline does not know or care how many sensors are real.

# 12. GitHub Commit

After Phase 2 passes all completion criteria, push to GitHub immediately:

cd ~/signgloves git add . git commit -m "Phase 2 complete: filter + normalize pipeline" git push

This locks the working Phase 2 state as a permanent snapshot before Phase 3 begins. If anything breaks in Phase 3, the working Phase 2 code is recoverable instantly.

Final note: Phase 2 is the most important foundational phase after Phase 1. Every piece of data that enters the ML model passes through the filter and calibration code written here. Bad filtering or bad calibration produces a bad dataset, which produces a bad model — with no obvious sign that anything went wrong. The time invested in understanding and testing Phase 2 correctly pays off in every phase that follows.