Smart Glove Dataset Studio

Phase 7 — Dataset Analysis Tools + Pipeline Debugging

AI Sign-to-Speech Smart Glove System | April 2026

Developer: Anoop B A

Repository: github.com/ANOOP222004/signgloves (Private)

Status: COMPLETE | Next Phase: Phase 8 — ML Export System

# 1. Phase 7 Overview

Phase 7 adds the Dataset Analysis Tools to the Smart Glove Dataset Studio — a Python desktop application for recording, managing, and exporting labeled gesture datasets for Indian Sign Language (ISL) recognition.

Phase 7 has two distinct parts:

- Part A — Critical pipeline debugging: Fixed a 4-8 Hz frame rate problem caused by two independent bugs discovered through systematic diagnosis using a standalone measurement script.

- Part B — Dataset Analysis Tools: Built the DATASET tab with sample balance tracking, gesture overlay visualization, outlier detection, feature statistics, speed tagging, and profile-based storage.

| Property | Value |
| --- | --- |
| Phase | Phase 7 — Dataset Analysis Tools + Pipeline Debugging |
| Status | COMPLETE |
| Frame Rate (before) | 4-10 Hz (broken — firmware sends 30 Hz perfectly) |
| Frame Rate (after) | 30-31 Hz (verified by diagnostic measurement) |
| New Files | dataset/dataset_analyzer.py, ui/analysis_tab.py |
| Modified Files | communication/serial_thread.py, communication/packet_parser.py, visualization/hand_skeleton.py, ui/recorder_panel.py, ui/main_window.py, dataset/dataset_manager.py, config.py, main.py |
| Hardware Required | Both gloves assembled and working (completed in Phase 6) |
| Next Phase | Phase 8 — ML Export System (blocked on dataset collection) |

# 2. The Debugging Journey — How We Found the Real Cause

This section is the most important part of Phase 7 for teammates to understand. The 4-10 Hz bug took multiple sessions to diagnose correctly because symptoms pointed in the wrong direction each time. The correct approach — measuring before fixing — ultimately solved it in one session.

## 2.1 The Problem

The Smart Glove Dataset Studio was showing RATE: 4-10 Hz in the status bar. At 30 Hz the system should capture 60 frames (2 seconds) per gesture recording. At 10 Hz each recording attempt was failing with "X frame(s) dropped during recording — please try again."

Dataset collection was completely impossible. Every recording attempt was discarded.

## 2.2 Wrong Hypotheses (What We Tried First)

Several fixes were attempted based on plausible guesses — none worked:

- Wrong port selected: ttyUSB0 (slave) vs ttyACM0 (master). Fixed by auto-detecting 'USB Single Serial' description. Did not fix frame rate.

- Firmware startup delay: 5-second 'Starting...' delay in setup() causing parser to receive noise. Removed delay. Did not fix frame rate.

- Python sleep too short: increased from 2s to 10s to wait for firmware. Did not fix frame rate.

- Serial timeout too long: changed readline() timeout from 1s to 0.1s. Did not fix frame rate.

- OS buffer: tried stty and setserial low_latency. Did not fix frame rate.

Each of these was a reasonable guess based on the symptoms — but guessing without measuring is inefficient.

## 2.3 The Diagnostic Script — Measure Before Fixing

A standalone serial_diagnostic.py script was written that does exactly one thing: open the serial port and count how many F, packets arrive per second for 10 seconds, with no Qt, no processing pipeline, no threading — just raw measurement.

Results on /dev/ttyACM0 (master port):

F, packet rate:   30.4 packets/sec

19 fields per packet — all valid

Zero noise lines after first packet at t=0.01s

This proved three things instantly:

- The firmware is perfect. It sends exactly 30 Hz.

- The port selection is correct. ttyACM0 is the master.

- The problem is entirely inside the Python pipeline. Something in the app is losing packets that the hardware delivers correctly.

The diagnostic script also ran against /dev/ttyUSB0 (slave) and confirmed 0 bytes — correct, slave only talks via ESP-NOW.

## 2.4 Root Cause — Two Independent Bugs

With the firmware confirmed good, the investigation shifted to comparing the diagnostic script (30 Hz) vs the full app (10 Hz). The only difference between them is what else is running alongside the serial thread. This pointed to two bugs:

### Bug 1 — packet_parser.py: Checksum Rejecting 70% of Valid Packets

The ESP32 computes the checksum using the full internal float value before rounding to 1 decimal place for the serial string. Example: the IMU pitch value is -48.123 internally. The firmware does int(-48.123 * 10) = -481 for the checksum, but sends "-48.1" in the packet.

Python was doing int(float("-48.1") * 10) = int(-481.0) = -481 for positive values, but for negative values floating point rounding causes int(-48.1 * 10) = int(-481.00000000006) = -481 in some cases and int(-480.99999999994) = -480 in others. The off-by-one mismatch caused approximately 70% of all valid packets to be silently discarded by the checksum gate.

Fix: Removed the checksum verification from parse_packet(). The field count (exactly 19 fields) and the 'F,' prefix are sufficient validation for a USB-CDC connection. The checksum formula mismatch was a firmware-Python floating point contract violation.

# REMOVED from packet_parser.py:

# if not verify_checksum(fields): return None

### Bug 2 — hand_skeleton.py: Rebuilding 40 OpenGL Meshes Every Frame

The HandSkeletonWidget._update_skeleton() method was called directly from on_frame() which fires at 30 Hz. Each call rebuilt 40 sphere meshes — running Python loops over 40 x 57 vertex iterations and issuing 40 setMeshData() OpenGL upload calls. This blocked the Qt main thread for approximately 200ms per call.

Since the Qt main thread handles both UI rendering and signal delivery, blocking it for 200ms meant the serial thread's signals were queued and delayed. The UI event loop ran at approximately 5 Hz instead of 30 Hz, causing the frame rate display to show 5-10 Hz even though frames were arriving at 30 Hz at the serial port.

Fix (same pattern as SignalPlotWidget which was already correct):

- on_frame() now only stores the latest frame values — no GL work, O(1) cost.

- A 25 Hz QTimer drives _update_skeleton() independently.

- Sphere meshes are pre-computed once in _precompute_sphere_cache() at startup.

- Per-frame update uses vectorized numpy (unit_verts + joint_pos) — one broadcast, no Python loop.

## 2.5 Result After Both Fixes

Measured pipeline throughput after fixes:

Frames in 5.0s: 151  =>  30.2 Hz

Frame drops during recording: zero. Dataset collection works correctly.

|  | Before Fix | After Fix |
| --- | --- | --- |
| Frame Rate | 4-10 Hz | 30.2 Hz |
| Packets accepted | ~30% (checksum bug) | ~100% |
| GL work per frame | 40 mesh rebuilds (200ms) | Zero (timer-driven) |
| Recording success | Always discarded | Clean 60-frame captures |

# 3. Part B — Dataset Analysis Tools

## 3.1 New Files

### dataset/dataset_analyzer.py

Read-only analysis engine — never modifies data, only reads it. Key methods:

- load_label(label) — returns ndarray shape (N, 60, 16) for all samples of a gesture

- load_label_with_filenames(label) — returns (ndarray, [filenames]) for deletion support

- compute_stats(label) — returns {'mean': (16,), 'std': (16,), 'n_samples': int}

- detect_outliers(label) — returns list of dicts with filename/feature/z_score/value/mean

- all_speed_counts() — returns {'HELLO': {'slow':0, 'medium':18, 'fast':0}, ...}

Outlier detection algorithm:

- Average each sample over its 60 frames → shape (N, 16)

- Compute z-score per feature across all N samples

- Flag samples where |z_score| > OUTLIER_Z_THRESHOLD (2.5)

- Skip features with std < 1e-6 (e.g. left IMU always 0.0 when no left-hand IMU)

- Requires minimum 3 samples for meaningful statistics

### ui/analysis_tab.py

Complete DATASET tab UI — fills the tab that was a stub in Phase 6. Layout:

- Top row: Gesture dropdown, Speed filter (ALL/SLOW/MEDIUM/FAST), profile label, ANALYZE button, REFRESH button

- Left panel: Sample counts list — all 10 gestures with counts colored red (<20), yellow (20-49), green (>=50). Shows per-speed breakdown. No bar chart — clean text list.

- Right panel: Single combined overlay graph — per sample, mean of all 10 finger channels (excludes IMU channels which are in degrees). Individual sample lines at alpha 0.3, colored by speed (slow=blue, medium=yellow, fast=green). Thick white mean line. Vertical dashed lines at zone boundaries for selected speed.

- Bottom: Outlier detection table with FILE/FEATURE/Z-SCORE/SAMPLE MEAN/GESTURE MEAN columns. DELETE SELECTED SAMPLE button.

- Feature statistics table: mean ± std for all 16 features, std color-coded green/yellow/gray by variation amount.

## 3.2 Speed Tagging System

Each recording is tagged with a speed (slow/medium/fast) selected by the user before pressing Record. The speed tag is embedded in the filename and affects how the gesture should be interpreted temporally.

Frame zone constants added to config.py:

SLOW_ZONES   = {'start': (0, 10),  'transition': (10, 50), 'end': (50, 60)}

MEDIUM_ZONES = {'start': (0, 8),   'transition': (8,  45), 'end': (45, 60)}

FAST_ZONES   = {'start': (0, 5),   'transition': (5,  30), 'end': (30, 60)}

The zones define where the meaningful gesture motion occurs. During analysis, vertical dashed lines are drawn at zone boundaries. For ML training, the transition zone is where the BiLSTM should focus — the start/end zones are hold positions.

Filename format: LABEL_SPEED_NNN.npy (e.g. HELLO_medium_001.npy, STOP_fast_003.npy)

Backward compatibility: samples without speed tag in filename are treated as medium.

## 3.3 Profile-Based Dataset Storage

Dataset folders are now organized by calibration profile name:

data/dataset/<profile_name>/LABEL/LABEL_SPEED_NNN.npy

The active calibration profile (set in the Calibration tab) automatically determines which dataset folder is used for all operations: recording, analysis, export. Switching profile switches the entire dataset context.

Migration: on startup, if old-style folders (data/dataset/HELLO/, data/dataset/STOP/ etc.) are found, they are automatically moved to data/dataset/default/ so no existing recordings are lost.

## 3.4 Combined Hand Signal Line

The gesture overlay uses a single combined line per sample instead of separate right-hand and left-hand plots. The combination formula:

combined[frame] = mean(R_T, R_I, R_M, R_R, R_L, L_T, L_I, L_M, L_R, L_L)

IMU channels (pitch, roll, yaw) are excluded — they are in degrees (not 0.0-1.0) so mixing them into a bend mean would distort the visualization. The result is a single 0.0-1.0 line per sample showing overall hand closure pattern across 60 frames.

# 4. Additional Fixes This Phase

| File | Issue | Fix |
| --- | --- | --- |
| main.py | os.environ.setdefault() skipped if variable existed — OpenGL stayed broken | Changed to os.environ[] (hard assignment, always overwrites) |
| serial_thread.py | readline() with timeout=1 blocked thread during GIL contention | Changed to chunk reading with in_waiting + time.sleep(0.005) |
| main.py port detection | Auto-selected CP2102 (slave) instead of USB Single Serial (master) | MASTER_PORT_KEYWORDS updated to match ttyACM0 description |
| ui/main_window.py | Maximize button made window disappear (xcb_egl fullscreen = invisible) | changeEvent override converts WindowFullScreen to showMaximized() |

# 5. How to Use the Dataset Tab

This section explains the Dataset tab for teammates who will use it during recording sessions.

## 5.1 Workflow

- Step 1: Record 10+ samples of a gesture in the Record tab.

- Step 2: Switch to Dataset tab. The sample counts list updates automatically.

- Step 3: Select the gesture in the Gesture dropdown. Click ANALYZE.

- Step 4: Inspect the overlay — do all sample lines follow a similar shape? Wildly different lines are bad recordings.

- Step 5: Check Outlier Detection table. Click any flagged row, then DELETE SELECTED SAMPLE to remove it.

- Step 6: Check Feature Statistics — std dev should be green (>0.15) for fingers that move during the gesture.

- Step 7: Repeat until all 10 gestures reach 50+ samples (green count).

## 5.2 What Good Data Looks Like

- Sample lines in overlay are clustered closely together — same shape, slight variation.

- No outliers flagged, or outliers clearly look wrong (flat when others are dynamic).

- Finger std dev values are green for fingers involved in the gesture.

- Count is green (50+) for the gesture.

## 5.3 What Bad Data Looks Like

- Sample lines spread wildly across 0.0-1.0 with no consistent shape — recalibrate.

- All lines are flat at 0.5 — calibration range is too small (open/closed ADC values are too similar).

- High IMU z-scores in outlier table — glove moved suddenly during that recording.

## 5.4 Speed Selection Guidance

| Speed | When to use | Transition frames | End padding |
| --- | --- | --- | --- |
| SLOW | Learning the sign, deliberate motion | 10-50 | 50-60 |
| MEDIUM | Natural conversational speed | 8-45 | 45-60 |
| FAST | Fluent signer, rapid gesture | 5-30 | 30-60 |

Record a mix of speeds for each gesture. The BiLSTM model learns to recognize the gesture regardless of speed. Aim for at least 15 slow + 20 medium + 15 fast per gesture (total 50+).

# 6. Key Design Decisions

| Decision | Choice | Reason |
| --- | --- | --- |
| Checksum removal | Removed from parser | Float rounding mismatch between firmware and Python caused 70% packet loss. Field count + prefix sufficient for USB-CDC. |
| Skeleton timer decoupling | 25 Hz QTimer independent of frames | GL mesh rebuilds at 30 Hz blocked Qt main thread 200ms/call, causing 5 Hz apparent rate. |
| Combined hand line | Mean of 10 finger channels only | IMU values are in degrees not 0-1 — cannot be averaged with bend values without distortion. |
| Outlier threshold | Z-score > 2.5 | 2.5 standard deviations catches clearly abnormal samples without flagging natural variation. |
| Speed zones | Three fixed zone sets (slow/medium/fast) | All 10 ISL signs complete in under 2 seconds. Zone tagging gives BiLSTM temporal context about where the gesture motion occurs. |
| Profile-based storage | data/dataset/<profile>/ | Different users have different sensor ADC ranges. Mixing their datasets would train the model on inconsistent normalization. |

# 7. Updated File Structure (After Phase 7)

~/signgloves/

├── smart_dataset_studio/

│   ├── main.py                      ← fixed: force env vars, port detection

│   ├── config.py                    ← added: SLOW/MEDIUM/FAST_ZONES, SPEED_TAGS

│   ├── communication/

│   │   ├── serial_thread.py         ← fixed: chunk read with in_waiting

│   │   └── packet_parser.py         ← fixed: removed checksum gate

│   ├── dataset/

│   │   ├── dataset_manager.py       ← rewritten: profile-based paths, speed tags

│   │   ├── dataset_analyzer.py      ← NEW: analysis engine

│   ├── ui/

│   │   ├── analysis_tab.py          ← NEW: complete Dataset tab

│   │   ├── recorder_panel.py        ← added: speed selector widget

│   │   └── main_window.py           ← added: changeEvent maximize fix

│   └── visualization/

│       └── hand_skeleton.py         ← fixed: timer-decoupled GL updates

├── docs/

│   └── SmartGlove_Phase7_Documentation.docx  ← THIS FILE

└── CURRENT_STATUS.md               ← updated

# 8. Next Phase — Phase 8: ML Export System

Phase 8 is blocked on dataset collection. The app is fully working at 30 Hz. Before starting Phase 8:

- Recalibrate with full open/closed hand range (target: range values 200-500+ per finger)

- Ensure magnet mounts are stable before recording — unstable magnets create inconsistent ADC readings across sessions

- Record 50+ samples per gesture for all 10 ISL signs

- Use Dataset tab to verify quality and delete outliers

Phase 8 will build:

- dataset/ml_export.py — exports X.npy (N, 60, 16), y.npy (N,), label_map.json

- Fills the EXPORT tab (currently a stub)

- After export: train BiLSTM on PC with TensorFlow/Keras, convert to TFLite INT8, flash to ESP32-S3 N16R8

Target: 95%+ accuracy on 10-sign ISL vocabulary (HELLO, STOP, YES, NO, THANK YOU, SORRY, HELP, WATER, PLEASE, MORE).

# 9. Constants Reference (config.py — After Phase 7)

| Constant | Value | Purpose |
| --- | --- | --- |
| BAUD_RATE | 115200 | Serial communication speed |
| SAMPLE_RATE | 30 | Frames per second from firmware |
| WINDOW_SIZE | 60 | Frames per gesture sample (2 seconds) |
| FRAME_ID_MAX | 9999 | Max frame ID — modulo uses 10000, not 9999 |
| QUEUE_MAX_SIZE | 100 | Do NOT rename to QUEUE_SIZE |
| EMA_ALPHA | 0.25 | Exponential moving average smoothing |
| OUTLIER_Z_THRESHOLD | 2.5 | Z-score threshold for outlier flagging |
| TARGET_SAMPLES | 50 | Target samples per gesture (green threshold) |
| DEFAULT_SPEED | medium | Default speed tag when not specified |
| SLOW_ZONES | see config | start:(0,10) transition:(10,50) end:(50,60) |
| MEDIUM_ZONES | see config | start:(0,8) transition:(8,45) end:(45,60) |
| FAST_ZONES | see config | start:(0,5) transition:(5,30) end:(30,60) |

# 10. Rules for Next Claude Code Sessions

These rules must be followed in every future session working on this project:

- Never run python3 main.py from Claude Code — always run the app yourself in a separate terminal. The serial port must be owned by your terminal, not Claude Code's subprocess.

- Read CURRENT_STATUS.md and CLAUDE.md before any work.

- The checksum is removed from packet_parser.py — do not add it back. The firmware-Python float rounding contract is broken by design.

- The skeleton widget uses a 25 Hz QTimer — do not connect _update_skeleton() directly to frame_ready signal.

- Dataset path is always data/dataset/<profile_name>/ — never flat data/dataset/.

- Speed tag is always embedded in filename: LABEL_SPEED_NNN.npy.

- os.environ[] (hard assignment) not setdefault() for OpenGL env vars in main.py.

- Master port is /dev/ttyACM0 (USB Single Serial) — slave is /dev/ttyUSB0 (CP2102).

- Always verify modified Python files with: python3 -c "import ast; ast.parse(open('file.py').read()); print('OK')"