AI Sign-to-Speech

Smart Gloves System

Phase 3 — Sensor Dashboard UI

Calibration Wizard + Live Sensor Display + Multi-User Profiles

Who this document is for: Every team member. It explains what Phase 3 builds, why every decision was made, how the new files connect to the existing pipeline, and what the completed UI looks like. Read this before touching any Phase 3 code.

# 1. What Phase 3 Is and Why It Exists

After Phase 2, the pipeline was complete but invisible. Data flowed from the ESP32 through the serial thread, into the processing thread, through EMA filtering and calibration normalization — and then disappeared into print() statements in the terminal. There was no way to see the sensor data without reading a scrolling console.

Phase 3 goal: replace the terminal with a real desktop window. After Phase 3, you open the app, complete a GUI calibration wizard, and see all 16 sensor values updating live on screen as you move your fingers.

This is the first visual milestone of the project — the moment the Smart Glove Dataset Studio becomes an application rather than a script.

## What Phase 3 Builds

| File | What It Does | Why It Exists |
| --- | --- | --- |
| ui/main_window.py | PyQt5 main window with live sensor display | Terminal output replaced with real UI |
| ui/calibration_wizard.py | Step-by-step GUI calibration with multi-user profiles | Terminal calibration replaced with guided GUI |
| ui/__init__.py | Empty file marking ui/ as Python package | Required for 'from ui.main_window import' to work |
| processing/calibration.py | Updated with profile support (list, create, save per profile) | Calibration files now organised per user in named folders |
| main.py | Updated: QApplication, wizard, window, signal wiring | Entry point upgraded from console script to GUI app |

# 2. Thread Architecture — What Changed

The three-thread architecture from Phase 2 is unchanged. Phase 3 adds the final consumer — the UI — at the end of the signal chain.

| Thread | Class | Role | Communicates Via |
| --- | --- | --- | --- |
| Main Thread | — | PyQt5 UI rendering — windows, buttons, labels | Receives Qt Signals from Processing Thread |
| Serial Thread | threading.Thread | Reads serial port, parses packets | Pushes frame dicts into queue.Queue |
| Processing Thread | QThread | Filters, normalizes, detects drops | Consumes Queue, emits Qt Signals to UI |

Critical rule unchanged from Phase 2: the UI never touches the Queue directly. The processing thread owns the Queue. The UI receives data only via Qt Signals. Violating this causes race conditions.

The signal connection that makes Phase 3 work:

processing_thread.frame_ready.connect(window.on_frame_ready) processing_thread.status_message.connect(window.on_status_message)

These two lines replace the on_frame_received() and on_status_message() functions from Phase 2. The window's own methods become the slots. Same signal — different receiver.

# 3. QApplication vs QCoreApplication

Phase 2 used QCoreApplication. Phase 3 changes it to QApplication. This is a one-line change but it is critical — without it, every window and dialog crashes immediately.

| Class | What It Provides | When To Use |
| --- | --- | --- |
| QCoreApplication | Qt event loop, signals/slots, timers — no graphics | Console apps, background services, no windows |
| QApplication | Everything in QCoreApplication plus full widget rendering stack | Any app that shows a window or dialog |

Rule: use QApplication the moment any QWidget, QDialog, or QMainWindow is created. QCoreApplication will crash with 'Must construct a QApplication before a QWidget'.

# 4. ui/main_window.py — The Main Window

MainWindow is the central desktop window. It receives processed frames via Qt Signal and updates 16 QLabel widgets in real time.

## Layout Structure

QMainWindow └── central_widget (QWidget) └── root_layout (QHBoxLayout) ├── Sensor Values panel (stretch=2) ← live data ├── Recorder panel (stretch=1) ← stub, Phase 4 └── Dataset panel (stretch=1) ← stub, Phase 4

Stretch values give the sensor panel twice as much horizontal space as each stub panel. This reflects the priority: the live data display is what you look at most during dataset collection.

## Sensor Values Panel

Contains two QGroupBox widgets side by side — Right Hand and Left Hand. Each shows 5 finger rows and 3 IMU rows. Label references are stored in dicts at construction time:

self._finger_labels[hand][channel] ← QLabel showing 0.00–1.00 self._imu_labels[hand][channel] ← QLabel showing degrees

Why store references? on_frame_ready() is called 30 times per second. Searching the widget tree on every call would be expensive. Direct dict access is O(1) — essentially free at 30 Hz.

## Monospace Font on Value Labels

All value labels use QFont('Courier', 10). This is not aesthetic — it is functional. Proportional fonts have characters of different widths. When a value changes from '0.21' to '0.99', the label physically resizes and the layout shifts. At 30 Hz, this produces visible UI jitter. Monospace fonts give every digit the same width — the label is stable regardless of value.

## Frame Rate Tracking

The status bar shows live frame rate. The implementation uses a counter and a 1-second timer:

# In on_frame_ready() — called 30x per second: self._frame_count += 1 # O(1), no timestamps, no division # In _update_fps() — called once per second by QTimer: fps = self._frame_count self._frame_count = 0 self._fps_label.setText(f"{fps} Hz")

The alternative — timestamping every frame and computing rate per-frame — requires division 30 times per second and floating-point timestamp math. The counter approach reads the count once per second. The result is identical but far cheaper.

## Stub Panels

The Recorder and Dataset panels are empty QGroupBox widgets with placeholder text. Phase 4 fills in their internals. The three-panel layout contract — sensor left, recorder middle, dataset right — is locked in. Phase 4 does not restructure the window.

# 5. ui/calibration_wizard.py — The Calibration Wizard

The calibration wizard replaces the terminal-based run_console_calibration() function from Phase 2. It produces the same result — a CalibrationData object with min/max per finger channel — but through a guided GUI dialog with multi-user profile support.

## Why QDialog with exec_()

exec_() runs a nested Qt event loop and blocks the caller until accept() or reject() is called. main.py does not continue past wizard.run_wizard() until calibration is complete. This is the correct behaviour — nothing should happen until we have valid calibration data. The alternative (show()) is non-blocking and would require callbacks to pass the result back.

## Why a Worker Thread for Frame Collection

Collecting 30 frames takes approximately 1 second at 30 Hz. Running this loop directly in a button slot would block the Qt event loop for 1 second — the window goes white and unresponsive. FrameCollectorThread runs the collection in the background and emits the result via Qt Signal back to the dialog on the main thread.

class FrameCollectorThread(QThread): collection_done = pyqtSignal(object) # carries list of frames collection_error = pyqtSignal(str) # carries error message def run(self): frames = [] for _ in range(self.num_frames): frame = self.frame_queue.get(timeout=2.0) frames.append(frame) self.collection_done.emit(frames)

## Why the Wizard Reads the Queue Directly

During calibration, the ProcessingThread has not started yet. Calibration needs raw ADC values from the Queue — the open-hand and closed-hand min/max readings must be in raw ADC space, not normalized 0.0–1.0 space. The wizard reads from frame_queue directly, exactly as run_console_calibration() did in Phase 2.

## QStackedWidget — Four Pages

The wizard uses a QStackedWidget to manage four content views. Only one page is visible at a time. setCurrentIndex(n) swaps pages instantly without rebuilding the widget tree.

| Page Index | Content | Action Button |
| --- | --- | --- |
| 0 | Profile selector — dropdown + optional name input | Continue |
| 1 | Open hand instruction + collection status | Ready |
| 2 | Closed hand instruction + collection status | Ready |
| 3 | Done summary — profile name + saved file path | Finish |

## _wire_action_btn() — The Button Role Pattern

The action button changes its function at every step. Without explicit disconnection, Qt stacks connections — clicking the button fires every previously connected handler simultaneously. _wire_action_btn() disconnects before connecting:

def _wire_action_btn(self, slot): try: self._action_btn.clicked.disconnect() except (RuntimeError, TypeError): pass # no previous connection — safe self._action_btn.clicked.connect(slot)

Note: PyQt5 raises TypeError (not RuntimeError) when disconnecting a signal with no connections. Both are caught to handle different PyQt versions.

# 6. Multi-User Profile System

Phase 3 adds named user profiles to the calibration system. Each profile has its own folder under data/calibration/. Calibration files are stored inside the profile folder with date-stamped names.

## File Structure

data/calibration/ ├── anoop/ │ ├── calibration_20250318.json │ └── calibration_20250319.json └── teammate/ └── calibration_20250318.json

## Why Per-Profile Folders

Each team member has different hand geometry and sensor placement. A calibration file is valid only for the person who recorded it on the hardware they used. Separate folders prevent one person's calibration from overwriting another's on the same day. Date-stamped filenames inside the folder preserve history — Hall sensors drift with temperature and wear, so older calibrations are kept rather than overwritten.

## New Functions in calibration.py

| Function | Signature | What It Does |
| --- | --- | --- |
| list_profiles() | () -> list | Scans CALIBRATION_DIR for subdirectories, returns sorted names |
| create_profile() | (name: str) -> str | Creates data/calibration/<name>/ folder, returns path |
| save_calibration() | (data, profile_name: str) -> str | Saves JSON to profile folder with today's date |
| get_today_calibration_path() | (profile_name: str) -> str¦None | Checks if today's JSON exists for given profile |
| load_calibration() | (filepath: str) -> CalibrationData | Unchanged — takes full path, profile-agnostic |

load_calibration() is unchanged. It takes a full filepath and does not need to know about profiles. The caller (wizard) resolves the path. This is the correct separation of concerns.

## Startup Flow with Profiles

1. Wizard opens — dropdown shows existing profiles + '+ Create new profile' 2a. First run: only create-new option — input field visible immediately 2b. Subsequent runs: existing profiles listed alphabetically 3. User selects profile — today's calibration checked 4a. If today's cal exists: popup asks 'Use existing? Yes/No' 4b. Yes: load and skip wizard steps entirely 4c. No / not found: run open-hand + closed-hand steps 5. main.py receives (profile_name, CalibrationData) 6. Window title shows profile name: 'Smart Glove Dataset Studio — anoop'

# 7. main.py — What Changed in Phase 3

main.py was updated surgically. The startup sequence order is identical to Phase 2 — only the calibration and signal-connection steps changed.

| Aspect | Phase 2 | Phase 3 |
| --- | --- | --- |
| Qt class | QCoreApplication | QApplication |
| Calibration | run_console_calibration() — terminal countdown | CalibrationWizard — GUI dialog |
| Signal receiver | on_frame_received() — print function | window.on_frame_ready() — UI update |
| Status receiver | on_status_message() — logger | window.on_status_message() — status bar |
| Window | None | MainWindow shown after calibration |
| Profile | None | profile_name returned by wizard, shown in title |

## Critical: Connect Signals Before start()

# CORRECT — connect before start() processing_thread.frame_ready.connect(window.on_frame_ready) processing_thread.status_message.connect(window.on_status_message) processing_thread.start() # thread starts AFTER connections are live

If start() is called first, the thread may emit frame_ready before the slot is connected. The first few frames are silently lost. Connecting first guarantees no signal is ever missed. This is the correct pattern for all QThread usage in this project.

## Startup Sequence (Phase 3)

| Step | Action | Why This Order |
| --- | --- | --- |
| 1 | Create QApplication | Must exist before any QWidget |
| 2 | Install Ctrl+C handler + QTimer | Before event loop starts |
| 3 | select_port() — input() in terminal | Safe: serial not started yet |
| 4 | Start SerialThread | NO input() after this point |
| 5 | time.sleep(2.0) + drain_queue() | ESP32 stabilise + flush startup noise |
| 6 | CalibrationWizard.run_wizard() | Blocks until calibration complete |
| 7 | Create MainWindow | After calibration data exists |
| 8 | Connect signals then ProcessingThread.start() | Connect before start — no missed signals |
| 9 | window.show() + app.exec_() | Event loop runs until quit |
| 10 | Stop processing thread then serial thread | Processing first — clean Queue drain |

# 8. Bug Fixed During Phase 3 Testing

## TypeError on First Dialog Open

Symptom: app crashed immediately when the calibration wizard opened with:

TypeError: disconnect() failed between 'clicked' and all its connections Aborted (core dump)

Cause: _wire_action_btn() caught RuntimeError when disconnecting a signal with no previous connections. PyQt5 actually raises TypeError in this case — not RuntimeError. The wrong exception type was caught, so the error propagated and crashed.

Fix: catch both exception types:

except (RuntimeError, TypeError): pass # PyQt5 raises TypeError, PyQt6 raises RuntimeError

Root cause note: This is a PyQt5 version inconsistency. Catching both types is the correct defensive approach regardless of PyQt version.

## Profile Name Input Hidden on First Run

Symptom: on first run, the dropdown showed '+ Create new profile' but the name input field was not visible — no way to type a profile name.

Cause: the input field was set to setVisible(False) at construction, relying on currentTextChanged signal to show it. On first run, '+ Create new profile' was already the selected item when the dialog opened — no selection change occurred, so the signal never fired.

Fix: check the initial dropdown value at construction time and set visibility accordingly:

is_create_new = (self._profile_combo.currentText() == CREATE_NEW_OPTION) self._new_profile_input.setVisible(is_create_new)

# 9. Phase 3 Completion Criteria — All Passed

| Criterion | Result |
| --- | --- |
| App opens → calibration wizard appears | ✅ Passed |
| Profile creation works (new name, folder created) | ✅ Passed |
| Open hand + close hand steps complete successfully | ✅ Passed |
| Calibration saved to data/calibration/<profile>/<date>.json | ✅ Passed |
| Main window opens with profile name in title bar | ✅ Passed |
| Bend prototype finger → R_T value updates live on screen | ✅ Passed |
| Status bar shows ~30 Hz | ✅ Passed |
| Second run: existing profile shown in dropdown | ✅ Passed |
| Second run: today's calibration detected, use/redo asked | ✅ Passed |

# 10. What Phase 4 Builds Next

Phase 3 is the last phase before data collection begins. Phase 4 is the core purpose of the entire application.

| File | What It Does |
| --- | --- |
| recording/gesture_recorder.py | Captures exactly 60 frames per sample, handles frame drops |
| dataset/dataset_manager.py | Saves samples as data/dataset/<LABEL>/sample_NNN.csv |
| ui/recorder_panel.py | Fills in the Recorder stub in MainWindow |
| ui/dataset_panel.py | Fills in the Dataset stub in MainWindow |

Phase 4 is unblocked — it requires only the one-finger prototype hardware currently available.

Phase 4 note for next session: the Recorder and Dataset panels in main_window.py are currently stubs (placeholder QGroupBox). Phase 4 replaces their internals. The three-panel layout structure does not change.

# 11. Key Design Decisions — Phase 3

| Decision | Choice Made | Why |
| --- | --- | --- |
| Qt application class | QApplication | Required for any window/widget to render |
| Calibration UI style | QDialog with exec_() | Blocks caller until complete — correct for mandatory step |
| Frame collection in wizard | FrameCollectorThread (QThread) | 1-second collection blocks UI if run in button slot |
| Wizard reads Queue directly | Yes — raw ADC | ProcessingThread not started yet; calibration needs raw values |
| Profile storage | Subdirectory per profile | Scales cleanly; each person gets folder with multiple date files |
| FPS display | Counter + 1-second timer | O(1) per frame; timestamps + division not needed |
| Value label font | Courier monospace | Prevents UI jitter from label resizing at 30 Hz |
| Signal connection order | Connect before start() | Prevents race condition losing first few frames |
| Button disconnect pattern | catch (RuntimeError, TypeError) | PyQt5 raises TypeError; PyQt6 raises RuntimeError |
| Stub panels | Empty QGroupBox with placeholder | Layout contract locked; Phase 4 fills internals without restructure |